SdkApi Guidance Document

| Current version | V1.241202 |
| --- | --- |
| Update date | 2024-12-02​ |
| Document security level | [ ] Internal [ √ ] Public |
| File Status | [ Draft] [ √ ] Official Release [ Under Revision ] |

Overview

Android motherboard supported APIs and its explanation

Product Version

| Motherboard Model | Supported versions |
| --- | --- |
| AIoT3568 | Android 11 |
| AI3399C | Android 10 |
| AIoT3588 | Android 12 |
| T950 | Android 14 |
| T982 | Android 14 |
| T963 | Android 9 |
| AIoT3576 | Android 14 |

Reader

This document is primarily intended for the following engineers:

- Software Development Engineer

- Technical Support Engineer

- Salesperson

Revision history

| date | Version | Edit description | author | Review |
| --- | --- | --- | --- | --- |
| 2021-08-02​ | V0.01 | Initial version | David | David |
| 2021-12-17​ | V0.02 | - Change the file font: Chinese to KaiTi and English to Arial. - Add API Interface Classification | David | David |
| 2021-12-18​ | V0.03 | - Add SN Write, MAC Read/write interface - Add serial port read/write interface - Add speaker parameter read/write interface - Remove USB Port power supply interface - Correcting errors in the document | David | David |
| 2022-02-23​ | V0.04 | - Increase CAN interface | David | David |

|  |  | 2. Add backlight PWM interface |  |  |
| --- | --- | --- | --- | --- |
| 2022-03-03​ | V0.05 | - Add Flash Read/write offset address interface - Add Flash interface type Type Description | David | David |
| 2022-03-22​ | V0.06 | - Add lvds Resolution Support List Interface | David | David |
| 2022-05-09​ | V0.07 | - Add custom resolution timing interface - Add a custom MIPI command interface | David | David |
| 2022-05-20​ | V0.08 | - Add backlight parameter setting and acquisition interface - Add interface for adjusting the display of strip screens / cut-out screens | David | David |
| 2022-05-24​ | V0.09 | - Add ICCID , IMEI , and shell. command line mouth | David | David |
| 2022-05-28​ | V0.10 | - Correction to screen resolution setting interface parameters bright | David | David |
| 2022-06-07​ | V0.11 | - Add CTEI Read/write interface | David | David |
| 2022-10-18​ | V0.12 | - Update Ethernet MAC settings Interface Description | David | David |
| 2022-11-28​ | V0.13 | - Add MCU Communication interface | Liu Tianxin |  |
| 2022-11-28​ | V0.14 | - Add API Service unbinding interface - Display type added DP port | David |  |
| 2023-05-05​ | V0.15 | - Add emmc Lifetime reading interface | David |  |
| 2023-07-26​ | V0.16 | - Increase Scale interface | David |  |
| 2023-12-05​ | V0.17 | - Fixed an error in the screen orientation setting example. - Add CommonListener interface - Increase LN interface | David |  |
| 2024-02-02​ | V1.240202 | - Add Ethernet settings - Add hotspot AP set up - Add Bluetooth and Wi-Fi Mac​ Get | Liu Feng | Liu Feng |
| 2024-03-29​ | V1.240329 | - Add boot animation change - Add screen saturation, contrast, and hue settings. - Add OTA Upgrade Interface | Zhanglongtan |  |
| 2024-04-12​ | V1.240412 | - Add Uboot logo Replacement Interface - Add kernel logo Replacement Interface | Liu Feng |  |
| 2024-04-16​ | V1.240416 | - Added interface for modifying boot animation sound | Li Lianfang |  |
| 2024-06-06​ | V1.240606 | - Add a mutually exclusive timed power on/off switch, with the timer set to date (year, month, day). Time | Liu Feng | Liu Feng |
| 2024-07-09​ | V1.240709 | - Add an interface for setting and acquiring signal brightness. | Liu Feng | Liu Feng |
| 2024-11-11​ | V1.241111 | - Add setting to specify CAN Device baud rate | Liu Feng | Liu Feng |
| 2024-12-02​ | V1.241202 | - Remove some external interfaces | Liu Feng | Liu Feng |

Table of contents

Table of contents

- initialization API 	9

  - void newInstance(Context context) 	9

Description: Instantiation SdkApi 	9

  - SdkApi getInstance() 	9

Description: Get SDK API Example 	9

  - boolean getServiceConnectionStatus() 	9

Description: SDK Service connection status 	9

  - void unBindApiSdkService() 	10

Description: Unbind SDK Service 	10

- Version information API 	10

  - String getApiVersion() 	10

Description: Get API Jar Package version 	10

  - String getApiSdkServiceVersion() 	10

Description: Get API SDK Service version 	10

  - String getFirmwareVersion() 	10

Description: Obtain motherboard firmware version 	10

  - String getChipName() 	10

Description: Retrieve motherboard main chipset name 	10

  - String getSerialNumber() 	11

Description: Retrieve motherboard serial number 	11

  - String getDDRSize() 	11

Description: Get motherboard memory size 	11

  - String getEMMCSize() 	11

Description: Get motherboard storage size 	11

  - void getMcuVersion(ResultCallback<ResultInfo<String>> cb) 	11

Description: MCU Version 	11

  - void setLcdDensity(int density) 	11

Description: Set system screen density 	to 11

  - int getLcdDensity() 	12

Description: Get system screen density 	12

  - void setProperty(String key, String value) 	12

Description: System settings prop property 	12

  - String getProperty(String key, String default) 	12

Description: Obtaining the system prop property 	12

  - boolean setSerialNumber(String (sn) 	Error! Bookmark not defined.

Description: Setting up the motherboard sn Serial number 	error! Bookmark not defined .

  - boolean setEthMac(String ethMac) 	Error! Bookmark not defined.

Description: Configure motherboard Ethernet MAC Incorrect address ! Bookmark not defined 	.

  - String getEthMac() 	12

Description: Obtain motherboard Ethernet MAC Address 	12

- Display related information API 	12

  - boolean setScreenRotate(int screen, int rotate) 	13

Description: Set screen orientation 	13

  - void getScreenRotate(int screen, ResultCallback<ResultInfo<Integer>> cb) 	13

Description: Get screen orientation 	13

  - boolean setScreenOnoff(int screen, boolean (onoff) 	13

Description: Set screen switch 	13

  - void getScreenOnoff(int screen, ResultCallback<ResultInfo<Boolean>> cb) 	14

Description: Get screen switch 	14

  - boolean setBrightness(int screen, int brightness) 	14

Description: Set screen brightness 	to 14.

  - void getBrightness(int screen, ResultCallback<ResultInfo<Integer>> cb) 	14

Description: Get screen brightness 	14

  - boolean setBacklightInverse(int screen, boolean (inverse) 	15

Description: Set backlight inversion to 	15.

  - void getBacklightInverse(int screen, ResultCallback<ResultInfo<Boolean>> cb) 15

Description: Obtain backlight inversion 	15

  - boolean setBacklightPWM(int screen, int min, int max) 	15

Description: Setting the backlight PWM Maximum and minimum values : 	15

  - void getBacklightPWM(int screen, ResultCallback<ResultInfo<String>> cb) 	15

Description: Obtain backlight PWM Maximum and minimum values : 	16

  - boolean setResolution(int screen, String resolution) 	16

Description: Set screen resolution 	to 16.

  - void getResolution(int screen, ResultCallback<ResultInfo<String>> cb) 	16

Description: Get screen resolution 	16

  - boolean setLvdsMode(int screen, int mode) 	16

Description: Settings LVDS mode 	16

  - void getLvdsMode(int screen, ResultCallback<ResultInfo<Integer>> cb) 	17

Description: Get LVDS mode 	17

  - boolean setLvdsSwap(int screen, int swap) 	17

Description: Settings lvds Even-ODD Exchange 	17

  - void getLvdsSwap(int screen, ResultCallback<ResultInfo<Integer>> cb) 	17

Description: Get lvds Even-ODD Exchange 	17

  - boolean setLvdsChannel(int screen, int ch) 	18

Description: Settings LVDS channel 	18

  - void getLvdsChannel(int screen, ResultCallback<ResultInfo<Integer>> cb) 	18

Description: Get LVDS channel 	18

  - String getSupportedResolutionsList(int (screen) 	18

Description: Retrieves a list of supported resolutions 	18

  - boolean setCustomTiming(CustomTiming timing) 	19

Description: Set a custom resolution Timing 	19

  - boolean setCustomMipiCmds(int[] cmdsBuf, int cmdsLen) 	19

Description: Custom settings mipi Instruction 	19

  - int setBacklightParams(int screen, int type, int typeValue) 	19

Description: Set backlight parameter 	to 19

  - int getBacklightParams(int screen, int type, 	20

ResultCallback<ResultInfo<Integer>> cb) 	20

Description: Obtain backlight parameters 	20

  - int setBacklightParams(int screen, int type, int typeValue) 	20

Description: Set backlight parameter 	to 20.

  - boolean setScreenGraphicDisplay(int scale, int x_pos, int y_pos, int width, int hight) 	20

Description: Sets the screen display size to adapt to strip screens / cut-out screens 	20.

  - void getScreenGraphicDisplay(ResultCallback<ResultInfo<String>> cb) 	21

Description: Detects screen display position, adapts to strip screens / cut-out screens 	21

- System Control API 	21

  - void deviceReboot() 	21

Description: Device restarted 	21

  - void shotScreen(String path, int screen, ResultCallback<Boolean> cb) 	21

Description: Screenshot 	21

  - void setStatusBarDisplay(boolean display) 	22

Description: Hide or show the navigation bar 	22

  - boolean getStatusBarDisplay() 	22

Description: Get navigation bar status 	22

  - void setDeviceShallowOnoff(boolean (onoff) 	22

Description: Device in normal standby mode 	22

  - boolean getDeviceShallowOnoff() 	22

Description: Obtain device standby status 	22

  - void setDeviceDeepStandby() 	22

Description: Device low power standby 	23

  - void setDeviceDeepStandby(int (time) 	23

Description: The device is in low-power, timed standby mode 	for 23 seconds.

  - boolean setFanOnoff(boolean (onoff) 	23

Description: Fan switch 	23

  - boolean getFanOnoff() 	23

Description: Get fan switch 	23

  - void silentInstallApk(String path, boolean isAuto, 	23

ResultCallback<ResultInfo<Boolean>> cb) 	23

Description: Silent installation App 	23

  - void silentUninstallApk(String packageName, 	24

ResultCallback<ResultInfo<Boolean>> cb) 	24

Description: Silent uninstallation App 	24

  - long getTPLastTouchTime() 	24

Description: Get the last touch time 	(24 seconds).

  - void formatStorageDevice(int type, ResultCallback<ResultInfo<Boolean>> cb) 24

Description: Format device 	24

  - boolean setBootVideoVolume(int volume) 	25

Description: Modify the boot animation volume 	to 25.

- Timed on/off API 	25

  - void setTimerSwitchOnoff(boolean (on/off) 	25

Description: Set timer switch 	25

  - boolean getTimerSwitchOnoff() 	25

Description: Get the status of the timer switch 	25

  - boolean setTimerOnTime(String (onTime) 	26

Description: Set a scheduled power-on time 	of 26.

  - void clearTimerOnTime() 	26

Description: Clear scheduled power-on time 	26

  - String getTimerOnTime() 	26

Description: Get the scheduled power-on time 	26

  - boolean setTimerOffTime(String offTime) 	26

Description: Set the timer to shut down 	in 26 seconds.

  - void clearTimerOffTime() 	26

Description: Clear the scheduled shutdown time 	26

  - String getTimerOffTime() 	27

Description: Get the scheduled shutdown time 	27

  - void setTimerStandbyMode(boolean (standby) 	27

Description: Set a timed shutdown mode 	27

  - boolean getTimerStandbyMode() 	27

Description: Get the scheduled shutdown mode 	27

  - boolean setTimerOnYmdTime() 	27

Description: Set a scheduled power-on time based on the year, month, and day. 	(27)

  - String getTimerOnYmdTime() 	27

Description: Retrieve scheduled power-on time (year, month, day) 	27

  - boolean setTimerOffYmdTime() 	28

Description: Set a timed shutdown time (year, month, day) 	28

  - String getTimerOffYmdTime() 	28

Description: Retrieve scheduled shutdown time (year, month, day) 	28

  - boolean setTimerType() 	28

Description: Set timed power on/off type 	28

  - boolean getTimerType() 	28

Description: Retrieve timed power on/off type 	28

- Feeding dogs API 	28

  - boolean setWatchdogOnoff(boolean (onoff) 	28

Description: Set watchdog switch 	29

  - boolean getWatchdogOnoff() 	29

Description: Get watchdog on/off status 	29

  - boolean setWatchdogTimeout(int (timeout) 	29

Description: Sets the watchdog timeout period, default value. 2 minutes 	29 seconds

  - boolean feedWatchdog() 	29

Description: Feeding the dog 	29

  - boolean watchdogSwitch(boolean (switch) 	29

Description: Feeding the dog switch 	29

- System time related API 	30

  - void setSystemTime(long (time) 	30

Description: Set system time 	to 30.

  - String getRtcTime() 	30

Description: Get RTC Time 	30

  - void setAutoTimeOnoff(boolean (on/off) 	30

Description: Set automatic calibration time 	to 30.

  - boolean getAutoTimeOnoff() 	30

Description: Obtain the status of the automatic calibration time switch 	30.

- Hardware related API 	30

  - int getGpioInput(int group, int num) 	30

Description: Get GPIO input level 	31

  - boolean setGpioOutput(int group, int num, boolean level) 	31

Description: Settings GPIO output status 	31

  - void flashRead(int type, int len, ResultCallback<ResultInfo<Integer>> cb) 	31

Description: Read flash Data 	31

  - void flashWrite(int type, int len, int[] buffer, 	32

ResultCallback<ResultInfo<Boolean>> cb) 	32

Description: Write flash Data 	32

  - void setRunningLedOnoff(boolean onoff, 	32

ResultCallback<ResultInfo<Boolean>> cb) 	32

Description: Motherboard operation indicator light 	32

  - void getRunningLedOnoff(ResultCallback<ResultInfo<Boolean>> cb) 	32

Description: Obtain the motherboard's operating indicator light 	32.

  - void setKeyBoardLedROnoff(boolean onoff, 	33

ResultCallback<ResultInfo<Boolean>> cb) 	33

Description: Keypad interface red indicator light switch 	33

  - void getKeyBoardLedROnoff(ResultCallback<ResultInfo<Boolean>> cb) 	33

Description: Obtain the red indicator light switch 	33 from the button panel interface.

  - void setKeyBoardLedGOnoff(boolean onoff, 	33

ResultCallback<ResultInfo<Boolean>> cb) 	33

Description: Keypad interface red indicator light switch 	33

  - void getKeyBoardLedGOnoff(ResultCallback<ResultInfo<Boolean>> cb) 	33

Description: Obtain the red indicator light switch 	33 from the button panel interface.

  - boolean setUsbPortPowerOnoff(int usbPort, boolean (on/off) 	34

Description: Settings USB Port power switch 	34

  - int getUsbPortPowerOnoff(int usbPort) 	34

Description: Get USB Port power status 	34

  - boolean setSpeakerConfigsSave(boolean status, int power int resistance) 	34

Description: Set speaker power parameter 	to 34

  - void getSpeakerConfigs(ResultCallback<ResultInfo<Integer>> cb) 	34

Description: Obtain speaker power parameters 	34

  - int openSerial(String serialName, int baud) 	35

Description: Open serial port device 	35

  - int closeSerial(int serial) 	35

Description: Close serial port device 	35

  - int readSerial(int serial, byte[] buf, int len) 	35

Description: Read serial port data 	35

  - int writeSerial(int serial, byte[] buf, int len) 	35

Description: Write serial port data 	35

  - int canBusConfigUp(int bitrate) 	36

Description: Configure and open can Bus 	36

  - int canOpen(int canId) 	36

Description: Open can Equipment 	36

  - int canSend(int can, int frameFlag, int frameId, byte[] canData, int len) 	36

Description: Send can Frame data 	36

  - int canRecv(int can, int[] canInfo) 	36

Description: Get can Frame data 	36

  - int canClose(int can) 	37

Description: Close can Equipment 	37

  - void flashReadAddr(int type, int addr, int len, 	37

ResultCallback<ResultInfo<Integer>> cb) 	37

Description: Read flash Data 	37

  - void flashWriteAddr(int type, int addr, int len, int[] buffer, 	37

ResultCallback<ResultInfo<Boolean>> cb) 	37

Description: Write flash Data 	37

  - int mcuRead(int reg, byte[] buf, int len 38	​

Description: Read MCU Register data 	38

  - int mcuWrite(int reg, byte[] buf, int len 38	​

Description: Modification MCU Register data 	38

  - int canBusConfigUpByID(int id, int bitrate) 	38

Description: Set the specified can The device's baud rate 	is 38.

- Other related API 	38

  - String getSimICCID() 	39

Description: Get Sim Card ICCID No. 	39

  - String getModemIMEI() 	39

Description: Get IMEI number 	39

  - void execShellCommand(String cmd) 	39

Description: Execution shell Command 	39

  - String getCTEI() 	39

Description: Get CTEI No. 	39

  - int setCTEI(String Error 	! Bookmark not defined.

Description: Settings CTEI Error 	! Bookmark not defined .

  - String getEmmcLeftTime() 	39

Description: Read emmc Lifespan 	39

- Scale Related API 	39

  - int registerScaleListener(int appId, ScaleListener listener) 	40

Description: Registration scale monitor listener 	40

  - int unregisterScaleListener(int appId, ScaleListener listener) 	40

Description: Cancellation scale monitor listener 	40

  - int setScaleEvent(int appId, int actionId, String strParam, int[] dataArray, 	40

int arrayLen) 	40

Description: Settings scale execution event 	40

- CommonEvent Related API 	41

  - int registerCommonListener(int appId, ISdkCommonListener listener) 	41

Description: Registration Common monitor listener 	41

  - int unregisterCommonListener(int appId, ISdkCommonListener listener) 	42

Description: Cancellation Common monitor listener 	42

  - int setCommonEvent(int appId, int actionId, String strParam, int[] dataArray, 	42

int arrayLen) 	42

Description: Settings Common Execution Event 	42

API Detailed description

- Initialize API

  - void newInstance(Context context)

Description: Instantiate SDK API

| type | describe |
| --- | --- |
| parameter | context : context |
| Return value |  |
| Example code | SdkApi.newInstance(this) |
| Remark |  |

  - SdkApi getInstance()

Description: Obtain an SDK API instance

| type | describe |
| --- | --- |
| parameter |  |
| Return value | SDK API Examples |
| Example code | SdkApi.getInstance() |
| Remark |  |

  - boolean getServiceConnectionStatus()

Description: SDK Service connection status

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Connection successful false : Connection failed |
| Example code | SdkApi.getInstance().getServiceConnectionStatus() |
| Remark |  |

  - void unBindApiSdkService()

Description: Unbind SDK Serve

| type | describe |
| --- | --- |
| parameter |  |
| Return value |  |
| Example code | SdkApi.getInstance().unBindApiSdkService() |
| Remark | After unbinding, you need to re-initialize. |

- Version Information API

API call method: SdkApi.getInstance().DeviceInfo(). API

  - String getApiVersion()

Description: Get API Jar Package version

| type | describe |
| --- | --- |
| parameter |  |
| Return value | API Jar Package version |
| Example code | SdkApi.getInstance().DeviceInfo().getApiVersion() |
| Remark |  |

  - String getApiSdkServiceVersion()

Description: Get API SDK Service Version

| type | describe |
| --- | --- |
| parameter |  |
| Return value | SDK Service Version |
| Example code | SdkApi.getInstance().DeviceInfo().getApiSdkServiceVersion() |
| Remark |  |

  - String getFirmwareVersion()

Description: Obtain the motherboard firmware version

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Motherboard firmware version |
| Example code | SdkApi.getInstance().DeviceInfo().getFirmwareVersion() |
| Remark |  |

  - String getChipName()

Description: Retrieve the motherboard main chipset name

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Motherboard main chip name |
| Example code | String chip = SdkApi.getInstance().DeviceInfo().getChipName() |
| Remark |  |

  - String getSerialNumber()

Description: Retrieve motherboard serial number

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Motherboard serial number |
| Example code | String sn = SdkApi.getInstance().DeviceInfo().getSerialNumber() |
| Remark | It reads vendor directly. Information burned |

  - String getDDRSize()

Description: Get motherboard memory size

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Motherboard memory size |
| Example code | String ddrSize = SdkApi.getInstance().DeviceInfo().getDDRSize() |
| Remark |  |

  - String getEMMCSize()

Description: Get motherboard storage size

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Motherboard storage size |
| Example code | String ddrSize = SdkApi.getInstance().DeviceInfo().getEMMCSize() |
| Remark | This interface is not supported; its functionality is not implemented. |

  - void getMcuVersion(ResultCallback<ResultInfo<String>> cb)

Description: MCU Version

| type | describe |
| --- | --- |
| parameter | cb : API callback interface |
| Return value | Callback val : mcu Version number |
| Example code | SdkApi.getInstance().DeviceInfo().getMcuVersion( new ResultCallback<ResultInfo<String>>() { @Override public void onMsg(String val) { } }) |
| Remark | Depending on the motherboard configuration, some motherboards do not have an MCU and do not support this interface. |

  - void setLcdDensity(int density)

Description: Set system screen density

| type | describe |
| --- | --- |
| parameter | density : Screen density, system default is 160. |
| Return value |  |

| Example code | SdkApi.getInstance().DeviceInfo().setLcdDensity(160) |
| --- | --- |
| Remark | A system restart is required for the changes to take effect. |

  - int getLcdDensity()

Description: Get system screen density

| type | describe |
| --- | --- |
| parameter |  |
| Return value | System screen density |
| Example code | int density = SdkApi.getInstance().DeviceInfo().getLcdDensity() |
| Remark |  |

  - void setProperty(String key, String value)

Description: Sets system prop properties

| type | describe |  |
| --- | --- | --- |
| parameter | key | prop key name |
|  | value | prop key corresponding value |
| Return value |  |  |
| Example code | SdkApi.getInstance().DeviceInfo().setProperty("persist.sys.timezone", (Asia/Shanghai) |  |
| Remark | the ro. attribute is not supported in Android 10/11. Because of SELinux This is a permissions issue; vendor involvement is recommended. beginning |  |

  - String getProperty(String key, String default)

Description: Retrieves system prop properties

| type | describe |  |
| --- | --- | --- |
| parameter | key | prop key name |
|  | default | prop key default value |
| Return value | System prop Parameter values corresponding to the attribute |  |
| Example code | String prop = SdkApi.getInstance().DeviceInfo().getProperty( "persist.sys.timezone", (Asia/Shanghai) |  |
| Remark |  |  |

  - String getEthMac()

Description: Obtain the motherboard's Ethernet MAC address address

| type | describe |  |
| --- | --- | --- |
| parameter |  |  |
| Return value | Ethernet MAC address, format: "0C63FC112233" |  |
| Example code | SdkApi.getInstance().DeviceInfo().getEthMac() |  |
| Remark | Macs are there on the board? When specifying the result, use @ as a separator, for example: 0 C 63 F C 1 122 33@ 0 C 63 F C 44 5 56 6 The first part corresponds to Ethernet 0 , and the second part corresponds to Ethernet 1. |  |

- Show relevant APIs

API call method: SdkApi.getInstance().Display(). interface

screen Display Interface ID The API Jar packages include definitions for LVDS, LVDS2, eDP, MIPI, VB1, and HDMI .

`ApiTool.Screen.ALL` : Indicates unified settings for all display interfaces on the motherboard. `ApiTool.Screen.LVDS` : Indicates the LVDS interface defined in the motherboard specification sheet. `ApiTool.Screen.LVDS2` : Indicates the LVDS2 interface defined in the motherboard specification sheet. Interfaces : `ApiTool.Screen.eDP` : Represents the eDP interface defined in the motherboard specification. `ApiTool.Screen.MIPI` : Represents the MIPI interface defined in the motherboard specification. `ApiTool.Screen.VB1`: Represents the VB1 interface defined in the motherboard specification. ` ApiTool.Screen.HDMI` : Represents the HDMI interface defined in the motherboard specification. `ApiTool.Screen.DP` : Represents the DP interface defined in the motherboard specification.

  - boolean setScreenRotate(int screen, int rotate)

Description: Set screen orientation

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | rotate | Direction, 0/90/180/270 |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setScreenRotate( ApiTool.Screen.ALL, 90) |  |
| Remark | A system restart is required for the changes to take effect. |  |

  - void getScreenRotate(int screen, ResultCallback<ResultInfo<Integer>> cb)

Description: Get screen orientation

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | API callback interface |
| Return value | Callback value : 0/90/180/270 |  |
| Example code | SdkApi.getInstance().Display().getScreenRotate(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setScreenOnoff(int screen, boolean (on/off)

Description: Set screen on/off

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | onoff | true : Open false : Close |
| Return value | true : Success |  |

|  | false : failure |
| --- | --- |
| Example code | boolean ret = SdkApi.getInstance().Display().setScreenOnoff( ApiTool.Screen.ALL, true) |
| Remark |  |

  - void getScreenOnoff(int screen, ResultCallback<ResultInfo<Boolean>> cb)

Description: Get screen switch

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | API callback interface |
| Return value | Callback val : true : on, false : off |  |
| Example code | SdkApi.getInstance().Display().getScreenOnoff(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Boolean>>() { @Override public void onMsg(Boolean val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setBrightness(int screen, int brightness)

Description: Set screen brightness

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | brightness | Screen brightness, range 0-100 |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setBrightness( ApiTool.Screen.ALL, 100) |  |
| Remark |  |  |

  - void getBrightness(int screen, ResultCallback<ResultInfo<Integer>> cb)

Description: Get screen brightness

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | API callback interface |
| Return value | Callback value : Screen brightness 0-100 |  |
| Example code | SdkApi.getInstance().Display().getBrightness(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setBacklightInverse(int screen, boolean inverse)

Description: Set backlight inversion

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | inverse | Backlight adjustment reverse, default is false. false : Backlight voltage 0-3.3/5V Or PWM Duty cycle 0-100 change true : Backlight voltage 3.3/5-0V Or PWM Duty cycle 100-0 change |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setBacklightInverse( ApiTool.Screen.ALL, false) |  |
| Remark | It is recommended to use the new interface setBacklightParams |  |

  - void getBacklightInverse(int screen, ResultCallback<ResultInfo<Boolean>> cb)

Description: Obtain backlight inversion

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | API callback interface |
| Return value | Callback val : true : on, false : off |  |
| Example code | SdkApi.getInstance().Display().getBacklightInverse( ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Boolean>>() { @Override public void onMsg(Boolean val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. It is recommended to use the new interface getBacklightParams |  |

  - boolean setBacklightPWM(int screen, int min, int max)

Description: Setting the backlight PWM Maximum and minimum values

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | min | Range 0-100​ |
|  | max | Range 0-100​ |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setBacklightPWM( ApiTool.Screen.ALL, 0, 0) |  |
| Remark | The setting will not take effect when the minimum value is greater than the maximum value. It is recommended to use the new interface setBacklightParams |  |

  - void getBacklightPWM(int screen, ResultCallback<ResultInfo<String>> cb)

Description: Obtaining backlight PWM Maximum and minimum values

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | The callback returns a string, with the minimum value first and the maximum value second. For example, " 10,100 " means that the minimum value is 10 and the maximum value is 100. |
| Return value |  |  |
| Example code | SdkApi.getInstance().Display().getBacklightPWM(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<String>>() { @Override public void onMsg(String val) { } }) |  |
| Remark | It is recommended to use the new interface getBacklightParams |  |

  - boolean setResolution(int screen, String resolution)

Description: Set screen resolution

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | resolution | Resolution format "1920x1080" Note: screen For HDMI When doing so, you need to add " @framerate ". For example: lvds : " 1920x1980 " , hdmi : " 3840x2160@25 ". |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setResolution( ApiTool.Screen.LVDS, "1920x1080") |  |
| Remark | Note that a restart is required for the changes to take effect. |  |

  - void getResolution(int screen, ResultCallback<ResultInfo<String>> cb)

Description: Get screen resolution

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | cb | API callback interface |
| Return value | Callback value : Resolution, formatted as "1920x1080@60" |  |
| Example code | SdkApi.getInstance().Display().getResolution(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<String>>() { @Override public void onMsg(String val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setLvdsMode(int screen, int mode)

Description: Setting LVDS mode

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) (Interface 1) , other interfaces are not supported. |
|  | mode | 0 : vesa 1 : jeida |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setLvdsMode( ApiTool.Screen.ALL, 0) |  |
| Remark |  |  |

  - void getLvdsMode(int screen, ResultCallback<ResultInfo<Integer>> cb)

Description: Get LVDS mode

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) (Interface 1) , other interfaces are not supported. |
|  | cb | API callback interface |
| Return value | Callback val : 0 : vesa , 1 : jeida |  |
| Example code | SdkApi.getInstance().Display().getLvdsMode(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setLvdsSwap(int screen, int swap)

Description: Setting up LVDS Even-ODD exchange

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) (Interface 1) , other interfaces are not supported. |
|  | swap | 0 : no swap 1 : swap |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setLvdsSwap( ApiTool.Screen.ALL, 0) |  |
| Remark |  |  |

  - void getLvdsSwap(int screen, ResultCallback<ResultInfo<Integer>> cb)

Description: Get lvds Even-ODD exchange

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) |

|  |  | (Interface 1) , other interfaces are not supported. |
| --- | --- | --- |
|  | cb | API callback interface |
| Return value | Callback val : 0 : no swap , 1 : swap |  |
| Example code | SdkApi.getInstance().Display().getLvdsSwap(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - boolean setLvdsChannel(int screen, int ch)

Description: Setting up an LVDS channel

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) (Interface 1) , other interfaces are not supported. |
|  | swap | 0 : signal 1 : dual |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display().setLvdsChannel( ApiTool.Screen.ALL, 0) |  |
| Remark |  |  |

  - void getLvdsChannel(int screen, ResultCallback<ResultInfo<Integer>> cb)

Description: Obtain LVDS channel

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display interface ID , supports LVDS , LVDS2 , ALL ( only updates LVDS) (Interface 1) , other interfaces are not supported. |
|  | cb | API callback interface |
| Return value | Callback val : 0 : signal , 1 : dual |  |
| Example code | SdkApi.getInstance().Display().getLvdsChannel(ApiTool.Screen.ALL, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }) |  |
| Remark | If the motherboard has dual outputs, screen Fill in all When it returns the current state of the home screen, it shows the current home screen state. |  |

  - String getSupportedResolutionsList(int screen)

Description: Retrieves a list of supported resolutions.

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |

| Return value | Supported resolution list string, " 1920x1080@60, 1366x768@60 " If the supported list is empty, return " none ". |
| --- | --- |
| Example code | String resolutionsList = SdkApi.getInstance().Display().getSupportedResolutionsList( ApiTool.Screen.Lvds) |
| Remark | When ID ALL ( only returns LVDS) interface ) |

  - boolean setCustomTiming(CustomTiming timing)

Description: Setting a custom resolution timing

| type | describe |  |
| --- | --- | --- |
| parameter | timing | Custom resolution timing |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display.setCustomTiming(new CustomTiming(...)); |  |
| Remark | A restart is required for the changes to take effect after importing. |  |

  - boolean setCustomMipiCmds(int[] cmdsBuf, int cmdsLen)

Description: Setting up a custom MIPI instruction

| type | describe |  |
| --- | --- | --- |
| parameter | cmdsBuf | Custom MIPI Instruction cache |
|  | cmdsLen | Data length |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display.setCustomMipiCmds( cmdsBuf, cmdsLen); |  |
| Remark | A restart is required for the changes to take effect after importing. |  |

  - int setBacklightParams(int screen, int type, int typeValue)

Description: Set backlight parameters

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | type | Type: ApiTool.BacklightType |
|  | typeValue | type: different types corresponding value illustrate type_pwm_dc: 0: dc, 1: pwm type_inverse: 0: no inverse, 1: inverse type_max : 0-100 , recommended > min type_min: 0-100 , recommended <max; type_pwm_freq : 100-100000 , unit: Hz |
| Return value | 0 : Success -1 : Failure |  |
| Example code | int ret = SdkApi.getInstance().Display.setBacklightParams( screen, type, typeValue); |  |

| Remark |  |
| --- | --- |

  - int getBacklightParams(int screen, int type,

ResultCallback<ResultInfo<Integer>> cb)

Description: Obtain backlight parameters

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | type | Type: ApiTool.BacklightType |
| Return value | callback val : type: different types The corresponding return value illustrate type_pwm_dc: 0: dc, 1: pwm type_inverse: 0: no inverse, 1: inverse type_max : 0-100 , recommended > min type_min: 0-100 , recommended <max; type_pwm_freq : 100-100000 , unit: Hz |  |
| Example code | SdkApi.getInstance().Display.getBacklightParams( screen, type, new ResultCallback<ResultInfo<Integer>>() { @Override public void onMsg(Integer val) { } }); |  |
| Remark |  |  |

  - int setBacklightParams(int screen, int type, int typeValue)

Description: Set backlight parameters

| type | describe |  |
| --- | --- | --- |
| parameter | screen | Display Interface ID |
|  | type | Type: ApiTool.BacklightType |
|  | typeValue | type: different types corresponding value illustrate type_pwm_dc: 0: dc, 1: pwm type_inverse: 0: no inverse, 1: inverse type_max : 0-100 , recommended > min type_min: 0-100 , recommended <max; type_pwm_freq : 100-100000 , unit: Hz |
| Return value | 0 : Success -1 : Failure |  |
| Example code | int ret = SdkApi.getInstance().Display.setBacklightParams( screen, type, typeValue); |  |
| Remark |  |  |

  - boolean setScreenGraphicDisplay(int scale, int x_pos, int y_pos, int width, int hight)

Description: Sets the screen display size to adapt to strip screens / cut-out screens.

| type | describe |  |
| --- | --- | --- |
| parameter | scale | Zoom switch; 0 : Off, 1 : On . The default is on. |
|  | x_pos | Horizontal offset relative to the top left corner of the physical map |
|  | y_pos | Vertical offset relative to the top left corner of the physical map |
|  | width | Display screen width |
|  | hight | Display screen height |
| Return value | true : Success false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Display. setScreenGraphicDisplay (scale, x, y, width, height); |  |
| Remark | Restart after setting to take effect. |  |

  - void getScreenGraphicDisplay(ResultCallback<ResultInfo<String>> cb)

Description: Detects screen display position for adaptation to strip screens / cut-out screens.

| type | describe |  |
| --- | --- | --- |
| parameter | cb | API callback interface |
| Return value | Callback value , formatted as "scale,0,0,1920,1080" |  |
| Example code | SdkApi.getInstance().Display().getScreenGraphicDisplay ( new ResultCallback<ResultInfo< String >>() { @Override public void onMsg(String val) { } }) |  |
| Remark |  |  |

- System Control API

API call method: SdkApi.getInstance().SystemCtrl(). API

  - void deviceReboot()

Description: Device restart

| type | describe |
| --- | --- |
| parameter |  |
| Return value |  |
| Example code | SdkApi.getInstance().SystemCtrl().deviceReboot() |
| Remark |  |

  - void shotScreen(String path, int screen, ResultCallback<Boolean> cb)

Description: Screenshot

| type | describe |  |
| --- | --- | --- |
| parameter | path | Screenshot file path and name |
|  | screen | Primary and secondary screens, 0 : primary screen, 1 : secondary screen; default is 0. |

|  | cb | callback interface |
| --- | --- | --- |
| Return value | Callback val : true : success, false : failure |  |
| Example code | SdkApi.getInstance().SystemCtrl().shotScreen("/mnt/sdcard/1.png", 0 new ResultCallback<ResultInfo<Boolean>>() { }) |  |
| Remark | Screenshots can only be in PNG format , and currently only screenshots of the home screen are supported. |  |

  - void setStatusBarDisplay(boolean display)

Description: Hides or shows the navigation bar

| type | describe |  |
| --- | --- | --- |
| parameter | display | true : Navigation bar displayed false : Hide navigation bar |
| Return value |  |  |
| Example code | SdkApi.getInstance().SystemCtrl().setStatusBarDisplay(false) |  |
| Remark |  |  |

  - boolean getStatusBarDisplay()

Description: Get navigation bar status

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Navigation bar displayed false : Hide navigation bar |
| Example code | SdkApi.getInstance().SystemCtrl().getStatusBarDisplay() |
| Remark |  |

  - void setDeviceShallowOnoff(boolean (on/off)

Description: Device in normal standby mode

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : Power on; false : Power off |
| Return value |  |  |
| Example code | SdkApi.getInstance().SystemCtrl().setDeviceShallowOnoff(false) |  |
| Remark |  |  |

  - boolean getDeviceShallowOnoff()

Description: Obtain device standby status

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Power on false : Power off |
| Example code | SdkApi.getInstance().SystemCtrl().getDeviceShallowOnoff() |
| Remark |  |

  - void setDeviceDeepStandby()

Description: Device in low-power standby mode

| type | describe |
| --- | --- |
| parameter |  |
| Return value |  |
| Example code | SdkApi.getInstance().setDeviceDeepStandby() |
| Remark |  |

  - void setDeviceDeepStandby(int time)

Description: Device in low-power timed standby mode

| type | describe |  |
| --- | --- | --- |
| parameter | time | Time until next power-on, in minutes, range 1-10080 |
| Return value |  |  |
| Example code | 1 Turn on the machine after hours SdkApi.getInstance().SystemCtrl().setDeviceDeepStandby(1*60) |  |
| Remark |  |  |

  - boolean setFanOnoff(boolean (on/off)

Description: Fan switch

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : Power on false : Power off, default is off. |
| Return value |  |  |
| Example code | boolean ret = SdkApi.getInstance().SystemCtrl().setFanOnoff(true) |  |
| Remark |  |  |

  - boolean getFanOnoff()

Description: Get fan switch

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Power on false : Power off |
| Example code | boolean onoff = SdkApi.getInstance().SystemCtrl().getFanOnoff() |
| Remark |  |

  - void silentInstallApk(String path, boolean isAuto, ResultCallback<ResultInfo<Boolean>> cb)

Description: Silent App Installation

| type | describe |  |
| --- | --- | --- |
| parameter | path | apk File path and name |
|  | isAuto | true : Launch the app after installation. false : The app does not launch after installation. |
|  | cb | Callback |
| Return value | Callback val : true : success, false : failure |  |

| Example code | SdkApi.getInstance().SystemCtrl().silentInstallApk("/mnt/sdcard/1.apk" , false, new ResultCallback<ResultInfo<Boolean>>() { @Override public void onMsg(Boolean val) { } }) |
| --- | --- |
| Remark |  |

  - void silentUninstallApk(String packageName, ResultCallback<ResultInfo<Boolean>> cb)

Description: Silently uninstall the app

| type | describe |  |
| --- | --- | --- |
| parameter | packageName | Package Name |
|  | cb | Callback |
| Return value | Callback val : true : success, false : failure |  |
| Example code | SdkApi.getInstance().SystemCtrl().silentUninstallApk( "com.sw.testdemo", new ResultCallback<ResultInfo<Boolean>>() { @Override public void onMsg(Boolean val) { } }) |  |
| Remark |  |  |

  - long getTPLastTouchTime()

Description: Get the time of the last touch.

| type | describe |
| --- | --- |
| parameter |  |
| Return value | The system time of the last touch; if the system has not been touched, it returns 0. |
| Example code | long time = SdkApi.getInstance().SystemCtrl().getTPLastTouchTime() |
| Remark |  |

  - void formatStorageDevice(int type, ResultCallback<ResultInfo<Boolean>> cb)

Description: Formatting the device

| type | describe |  |
| --- | --- | --- |
| parameter | type | Format type: SDCard ( ApiTool.Format.SDCard ) Internal ( ApiTool.Format.Internal ) U Disk ( ApiTool.Format.Usb ) |
|  | cb | Callback |
| Return value | Callback val : true : success, false : failure |  |
| Example code | SdkApi.getInstance().SystemCtrl().getTPLastTouchTime(type, new ResultCallback<ResultInfo<Boolean>>() { @Override |  |

|  | public void onMsg(Boolean val) { } }) |
| --- | --- |
| Remark |  |

  - boolean setBootVideoVolume(int volume)

Description: Modify the boot animation volume

| type | describe |  |
| --- | --- | --- |
| parameter | volume | 0 <= volume <= 100 |
| Return value | Callback val : true : success, false : failure |  |
| Example code | SdkApi.getInstance().SystemCtrl().setBootVideoVolume(95); |  |
| Remark | Set the boot animation volume to 95%. |  |

- Timed power on/off API

API call method: SdkApi.getInstance().TimerSwitch(). API function description: Power on during the specified time period and power off outside the specified time period.

Example:

The machine is powered on at 9:00 AM and powered off at 10:00 AM . Operating hours are from 9:00 AM to 10:00 AM .

Power on at 10:00 AM , power off at 9:00 AM , equipment operating hours are 10:00 AM to 11:00 AM daily. Orders until 9:00 the next day

Power-on time = power-off time, invalid setting, maintain current state.

  - void setTimerSwitchOnoff(boolean (on/off)

Description: Set a timer switch

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | App Timer switch, off by default. |
| Return value |  |  |
| Example code | SdkApi.getInstance().TimerSwitch().setTimerSwitchOnoff(true) |  |
| Remark | - It is disabled by default after powering on or restarting the system. - After powering on or restarting the system 2 The timer starts after 2 minutes, so power on 2 The timer will not work within minutes. |  |
| Remark |  |  |

  - boolean getTimerSwitchOnoff()

Description: Get the status of the timer switch

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Open false : Close |
| Example code | SdkApi.getInstance().TimerSwitch().getTimerSwitchOnoff() |

| Remark |  |
| --- | --- |

  - boolean setTimerOnTime(String onTime)

Description: Set a scheduled power-on time

| type | describe |  |
| --- | --- | --- |
| parameter | onTime | Scheduled power-on time, format: "HH:mm" |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().TimerSwitch().setTimerOnTime("12:01") |  |
| Remark |  |  |

  - void clearTimerOnTime()

Description: Clear scheduled power-on time

| type | describe |
| --- | --- |
| parameter |  |
| Return value |  |
| Example code | SdkApi.getInstance().TimerSwitch().clearTimerOnTime() |
| Remark | The default time is "00:00" |

  - String getTimerOnTime()

Description: Get the scheduled power-on time

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Power-on time, format: "HH:mm" |
| Example code | SdkApi.getInstance().TimerSwitch().getTimerOnTime() |

  - boolean setTimerOffTime(String offTime)

Description: Set a timed shutdown time

| type | describe |  |
| --- | --- | --- |
| parameter | offTime | Scheduled shutdown time, format: "HH:mm" |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().TimerSwitch().setTimerOffTime("12:01") |  |
| Remark |  |  |

  - void clearTimerOffTime()

Description: Clear the scheduled shutdown time.

| type | describe |
| --- | --- |
| parameter |  |
| Return value |  |
| Example code | SdkApi.getInstance().TimerSwitch().clearTimerOffTime() |
| Remark | The default time is "00:00" |

  - String getTimerOffTime()

Description: Get the scheduled shutdown time

| type | describe |
| --- | --- |
| parameter |  |
| Return value | Shutdown time, format: "HH:mm" |
| Example code | SdkApi.getInstance().TimerSwitch().getTimerOffTime() |
| Remark |  |

  - void setTimerStandbyMode(boolean standby)

Description: Set a timed shutdown mode

| type | describe |  |
| --- | --- | --- |
| parameter | standby | false : Normal standby mode, this is the default mode. true : Low power standby |
| Return value |  |  |
| Example code | SdkApi.getInstance().TimerSwitch().setTimerStandbyMode(false) |  |
| Remark |  |  |

  - boolean getTimerStandbyMode()

Description: Get the scheduled shutdown mode

| type | describe |
| --- | --- |
| parameter |  |
| Return value | false : Normal standby true : Low power standby |
| Example code | SdkApi.getInstance().TimerSwitch().getTimerStandbyMode() |
| Remark |  |

  - boolean setTimerOnYmdTime ()

Description: Set a scheduled power-on time based on the given date.

| type | describe |
| --- | --- |
| parameter | String onTime |
| Return value | false : Setup failed true : Setting successful |
| Example code | SdkApi.getInstance().TimerSwitch(). setTimerOnYmdTime ("2024-06-06 06:06") |
| Remark | Input parameters are yyyy-MM-dd HH:mm Please use SimpleDateFormat |

  - String g etTimerOnYmdTime ()

Description: Retrieve scheduled power-on time (year, month, day).

| type | describe |
| --- | --- |
| parameter |  |
| Return value | String : 2024-06-06 06:06 |
| Example code | SdkApi.getInstance().TimerSwitch().g etTimerOnYmdTime () |
| Remark |  |

  - boolean setTimerOffYmdTime ()

Description: Set a scheduled shutdown time based on the given date and time.

| type | describe |
| --- | --- |
| parameter | String offTime |
| Return value | false : Setup failed true : Setting successful |
| Example code | SdkApi.getInstance().TimerSwitch(). setTimerOffYmdTime ("2024-06-06 06:06") |
| Remark | Input parameters are yyyy-MM-dd HH:mm Please use SimpleDateFormat |

  - String g etTimerOffYmdTime ()

Description: Retrieve scheduled shutdown time (year, month, day).

| type | describe |
| --- | --- |
| parameter |  |
| Return value | String : 2024-06-06 06:06 |
| Example code | SdkApi.getInstance().TimerSwitch().g etTimerOffYmdTime () |
| Remark |  |

  - boolean setTimerType ()

Description: Set the timed on/off type

| type | describe |
| --- | --- |
| parameter | boolen type |
| Return value | false : Setup failed true : Setting successful |
| Example code | SdkApi.getInstance().TimerSwitch(). setTimerType (true) |
| Remark | Set to true If the power is set to turn on/off on a specific date (year, month, day), then it will turn on/off on a specific weekday. |

  - boolean g etTimerType ()

Description: Retrieve timed power on/off type

| type | describe |
| --- | --- |
| parameter | boolen type |
| Return value | false : Scheduled on/off operation by week true : Scheduled power on/off on/off date |
| Example code | SdkApi.getInstance().TimerSwitch().g etTimerType () |
| Remark |  |

- APIs related to feeding dogs

API call method: SdkApi.getInstance().Watchdog(). API

Function: Watchdog reset, default SDK Feed the dog, App When feeding the dog, Sdk Launch App Dog feeding service, monitoring app

Feeding the dog

  - boolean setWatchdogOnoff(boolean (on/off)

Description: Setting up the watchdog switch

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | App Watchdog switch, off by default. |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().Watchdog().setWatchdogOnoff(true) |  |
| Remark |  |  |

  - boolean getWatchdogOnoff()

Description: Get watchdog on/off status

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Open false : Close |
| Example code | boolean ret = SdkApi.getInstance().Watchdog().getWatchdogOnoff() |
| Remark |  |

  - boolean setWatchdogTimeout(int timeout)

Description: Sets the watchdog timeout period, default is 2 minutes.

| type | describe |  |
| --- | --- | --- |
| parameter | timeout | App Watchdog timeout, default 2 seconds. Minutes, range 30 sec -5 minute, Unit: seconds |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().Watchdog().setWatchdogTimeout(60) |  |
| Remark |  |  |

  - boolean feedWatchdog()

Description: Feeding the dog

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Success false : failure |
| Example code | boolean ret = SdkApi.getInstance().Watchdog().feedWatchdog() |
| Remark |  |

  - boolean watchdogSwitch(boolean switch)

Description: Switching between feeding the dog

| type | describe |  |
| --- | --- | --- |
| parameter | switch | App Switching between feeding the dog and defaulting to feeding the dog via the system. true : App Feed the dog, false : System feed dog |
| Return value | true : Open false : Close |  |

| Example code | SdkApi.getInstance().watchdogSwitch(true) |
| --- | --- |
| Remark | This interface is not available. |

- System time related APIs

API call method: SdkApi.getInstance().SystemTime(). interface

  - void setSystemTime(long time)

Description: Set system time

| type | describe |  |
| --- | --- | --- |
| parameter | time | System Time |
| Return value | Motherboard firmware version |  |
| Example code | SdkApi.getInstance().SystemTime().setSystemTime(60*1000*1000) |  |
| Remark |  |  |

  - String getRtcTime()

Description: Obtain RTC time

| type | describe |
| --- | --- |
| parameter |  |
| Return value | RTC time, time format: "yyyy/MM/dd" hh:mm:ss" |
| Example code | String RTC = SdkApi.getInstance().SystemTime().getRtcTime() |
| Remark |  |

  - void setAutoTimeOnoff(boolean (on/off)

Description: Set automatic calibration time

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : On; false : Off |
| Return value |  |  |
| Example code | SdkApi.getInstance().SystemTime().setAutoTimeOnoff(true) |  |
| Remark |  |  |

  - boolean getAutoTimeOnoff()

Description: Obtain the status of the automatic calibration time switch.

| type | describe |
| --- | --- |
| parameter |  |
| Return value | true : Open false : Close |
| Example code | SdkApi.getInstance().SystemTime().getAutoTimeOnoff() |
| Remark |  |

- Hardware-related APIs

API call method: SdkApi.getInstance().Hardware(). interface

  - int getGpioInput(int group, int num)

Description: Obtain GPIO input level

| type | describe |  |
| --- | --- | --- |
| parameter | group | GPIO The group it belongs to; for example: GPIO0_B3 , group That is GPIO0 |
|  | num | The GPIO 's sequence number within the group; for example: GPIO0_B3 , num It's B3 |
| Return value | 1: High level 0: Low level -1 : Read error |  |
| Example code | GPIO0_B3: int level = SdkApi.getInstance().Hardware().getGpioInput( ApiTool.GpioPin.GPIO0, ApiTool.GpioPin.PB3); |  |
| Remark |  |  |

  - boolean setGpioOutput(int group, int num, boolean level)

Description: Set GPIO output status

| type | describe |  |
| --- | --- | --- |
| parameter | group | GPIO The group it belongs to; for example: GPIO0_B3 , group That is GPIO0 |
|  | num | The GPIO 's sequence number within the group; for example: GPIO0_B3 , num It's B3 |
|  | level | true : high level, false : low level |
| Return value | true : Success false : failure |  |
| Example code | GPIO0_B3: int level = SdkApi.getInstance().Hardware().setGpioOutput(ApiTool.GpioPin.GPIO0, ApiTool.GpioPin.PB3, true); |  |
| Remark |  |  |

  - void flashRead(int type, int len, ResultCallback<ResultInfo<Integer>> cb)

Description: Reading flash memory data

| type | describe |  |
| --- | --- | --- |
| parameter | type | Data type (HWID, (Authorization information, etc. ) ApiTool.FlashRWType kind |
|  | len | Length of data to be read |
|  | cb | Callback |
| Return value | Callback, val1 : the data read. val2 : The actual length read |  |
| Example code | SdkApi.getInstance().Hardware().flashRead( ApiTool.FlashRWType.HWID, 1, new ResultCallback<ResultInfo<Interger>>() { @Override public void onMsg(ResultInfo<Integer> val) { byte[] array = val.getArrayByte(); int len = val.getVal(); } }); |  |
| Remark | Read type The entire data block, typically a maximum of 1024. byte, some types Only a few words |  |

|  | Festival |
| --- | --- |

  - void flashWrite(int type, int len, int[] buffer, ResultCallback<ResultInfo<Boolean>> cb)

Description: Writing Flash data

| type | describe |  |
| --- | --- | --- |
| parameter | type | Data type (HWID, (Authorization information, etc. ) ApiTool.FlashRWType kind |
|  | len | Write data length |
|  | buffer | Write data cache |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().Hardware().flashWrite( ApiTool.FlashRWType.APP_INFO, 1, buffer new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark |  |  |

  - void setRunningLedOnoff(boolean onoff,

ResultCallback<ResultInfo<Boolean>> cb)

Description: Motherboard power indicator light

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : On, false : Off |
|  | cb | Callback, returns to the setting state |
| Return value | Callback to retrieve setting results: true : Success ; false : Failure |  |
| Example code | SdkApi.getInstance().Hardware().setRunningLedOnoff(true, new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Depending on the motherboard configuration, some motherboards may not have indicator lights, or the indicator lights may be determined by hardware circuitry. The default setting is true. |  |

  - void getRunningLedOnoff(ResultCallback<ResultInfo<Boolean>> cb)

Description: Obtain the motherboard's operating indicator light

| type | describe |  |
| --- | --- | --- |
| parameter | cb | Callback, returns to the setting state |
| Return value | Callback to retrieve status: true : On; false : Off |  |
| Example code | SdkApi.getInstance().Hardware().getRunningLedOnoff( new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Depending on the motherboard configuration, some motherboards may not have indicator lights, or the indicator lights may be determined by hardware circuitry. |  |

|  | The default setting is true. |
| --- | --- |

  - void setKeyBoardLedROnoff(boolean onoff, ResultCallback<ResultInfo<Boolean>> cb)

Description: Red indicator light switch for button panel interface

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : On; false : Off |
|  | cb | Callback |
| Return value | Callback to retrieve setting results: true : Success ; false : Failure |  |
| Example code | SdkApi.getInstance().Hardware().setKeyBoardLedROnoff(true, new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Based on the hardware, the default is LED. Common cathode design control |  |

  - void getKeyBoardLedROnoff(ResultCallback<ResultInfo<Boolean>> cb)

Description: Obtain the red indicator light switch from the button panel interface.

| type | describe |  |
| --- | --- | --- |
| parameter | cb | Callback |
| Return value | Callback to retrieve status: true : On; false : Off |  |
| Example code | SdkApi.getInstance().Hardware().getKeyBoardLedROnoff( new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Based on the hardware, the default is LED. Common cathode design control |  |

  - void setKeyBoardLedGOnoff(boolean onoff, ResultCallback<ResultInfo<Boolean>> cb)

Description: Red indicator light switch for button panel interface

| type | describe |  |
| --- | --- | --- |
| parameter | onoff | true : On; false : Off |
|  | cb | Callback |
| Return value | Callback to retrieve setting results: true : Success ; false : Failure |  |
| Example code | SdkApi.getInstance().Hardware().setKeyBoardLedGOnoff(true, new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Based on the hardware, the default is LED. Common cathode design control |  |

  - void getKeyBoardLedGOnoff(ResultCallback<ResultInfo<Boolean>> cb)

Description: Obtain the red indicator light switch from the button panel interface.

| type | describe |  |
| --- | --- | --- |
| parameter | cb | Callback |
| Return value | Callback to retrieve status: true : On; false : Off |  |
| Example code | SdkApi.getInstance().Hardware().getKeyBoardLedGOnoff( new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark | Based on the hardware, the default is LED. Common cathode design control |  |

  - boolean setUsbPortPowerOnoff(int usbPort, boolean (on/off)

Description: Setting up USB Port power switch

| type | describe |  |
| --- | --- | --- |
| parameter | usbPort | USB port index number |
|  | onoff | true : On; false : Off |
| Return value |  |  |
| Example code | SdkApi.getInstance().Hardware().setUsbPortPowerOnoff(port, true); |  |
| Remark | This interface is not available. Depending on the hardware, USB... Different port index numbers |  |

  - int getUsbPortPowerOnoff(int usbPort)

Description: Obtain USB Port power status

| type | describe |  |
| --- | --- | --- |
| parameter | usbPort | USB port index number |
| Return value | true : On, false : Off |  |
| Example code | SdkApi.getInstance().Hardware().getUsbPortPowerOnoff(port); |  |
| Remark | This interface is not available. Depending on the hardware, USB... Different port index numbers |  |

  - boolean setSpeakerConfigsSave(boolean status, int power int resistance)

Description: Setting speaker power parameters

| type | describe |  |
| --- | --- | --- |
| parameter | status | The speaker parameters are stored in the state: true : enabled, false : disabled, default DTS. |
|  | power | Speaker power: 1-10W |
|  | resistance | Speaker resistance: 2, 4 , 6, 8 Ω |
| Return value | true : success, false : failure |  |
| Example code | boolean ret = SdkApi.getInstance().Hardware(). setSpeakerConfigsSave(status, power resistance); |  |
| Remark |  |  |

  - void getSpeakerConfigs(ResultCallback<ResultInfo<Integer>> cb)

Description: Obtain speaker power parameters

| type | describe |  |
| --- | --- | --- |
| parameter | cb | Get speaker power callback |
| Return value | Retrieving the array via callback: |  |

|  | The first data is status . The second data point is the speaker power; the third data point is the speaker resistance. |
| --- | --- |
| Example code | SdkApi.getInstance().Hardware().getSpeakerConfigs( new ResultCallback<ResultInfo<Integer>>() { ... byte[] array = val.getArrayByte(); ... }); |
| Remark |  |

  - int openSerial(String serialName, int baud)

Description: Open the serial port device

| type | describe |  |
| --- | --- | --- |
| parameter | serialName | The serial port device name, for example, " /dev/ttyS0 " |
|  | baud | Serial port baud rate |
| Return value | Serial port device handle |  |
| Example code | int serial = SdkApi.getInstance().Hardware().openSerial(“/dev/ttyS0”, 115200); |  |
| Remark |  |  |

  - int closeSerial(int serial)

Description: Close serial port device

| type | describe |  |
| --- | --- | --- |
| parameter | serial | The handle obtained when opening the serial port |
| Return value | 0 : Closing successful; other : Closing failed. |  |
| Example code | SdkApi.getInstance().Hardware().closeSerial(serial); |  |
| Remark |  |  |

  - int readSerial(int serial, byte[] buf, int len)

Description: Read serial port data

| type | describe |  |
| --- | --- | --- |
| parameter | serial | Handle to the corresponding serial port |
|  | buf | Cache for saving read data |
|  | len | Length of data to be read |
| Return value | Actual data length read |  |
| Example code | int readLen = SdkApi.getInstance().Hardware().readSerial(serial, buf, len); |  |
| Remark |  |  |

  - int writeSerial(int serial, byte[] buf, int len)

Description: Write serial port data

| type | describe |
| --- | --- |

| parameter | serial | Handle to the corresponding serial port |
| --- | --- | --- |
|  | buf | Cache for written data |
|  | len | Length of data to be written |
| Return value | Actual length of data written |  |
| Example code | int writeLen = SdkApi.getInstance().Hardware().writeSerial(serial, buf, len); |  |
| Remark |  |  |

  - int canBusConfigUp(int bitrate)

Description: Configure and enable CAN bus

| type | describe |  |
| --- | --- | --- |
| parameter | bitrate | can Bus speed |
| Return value | 0 : Success, other : Failure |  |
| Example code | int ret = SdkApi.getInstance().Hardware().canBusConfigUp(250000); |  |
| Remark |  |  |

  - int canOpen(int canId)

Description: Open CAN equipment

| type | describe |  |
| --- | --- | --- |
| parameter | canId | can Bus ID |
| Return value | can Device handle |  |
| Example code | int ret = SdkApi.getInstance().Hardware().canOpen(0); |  |
| Remark | id The default value is 0. |  |

  - int canSend(int can, int frameFlag, int frameId, byte[] canData, int len)

Description: Send CAN Frame data

| type | describe |  |
| --- | --- | --- |
| parameter | can | Corresponding CAN device handle |
|  | frameFlag | Frame type: such as CAN_EFF_FLAG |
|  | frameId | Frame identifier |
|  | canData | Frame data |
|  | len | Frame data length, maximum 8 bytes |
| Return value | can Device handle |  |
| Example code | int ret = SdkApi.getInstance().Hardware().canSend(can,0,0x123,0x11,1); |  |
| Remark |  |  |

  - int canRecv(int can, int[] canInfo)

Description: Get CAN Frame data

| type | describe |  |
| --- | --- | --- |
| parameter | can | Corresponding CAN device handle |
|  | canInfo | The retrieved data, canInfo[0] is can_id, canInfo[1] is can_dlc |

|  |  | canInfo[2] is can_data Starting data |
| --- | --- | --- |
| Return value | The entire canInfo Data length |  |
| Example code | int[] canInfo = new int[16]; int ret = SdkApi.getInstance().Hardware().canRecv(can, canInfo); |  |
| Remark |  |  |

  - int canClose(int can)

Description: Disable CAN equipment

| type | describe |  |
| --- | --- | --- |
| parameter | can | Corresponding CAN device handle |
| Return value | 0 : Success, other : Failure |  |
| Example code | int ret = SdkApi.getInstance().Hardware().canClose(can); |  |
| Remark |  |  |

  - void flashReadAddr(int type, int addr, int len,

ResultCallback<ResultInfo<Integer>> cb)

Description: Reading flash memory data

| type | describe |  |
| --- | --- | --- |
| parameter | type | Data type (HWID, APP_INFO wait ) ApiTool.FlashRWType kind |
|  | addr | Read address |
|  | len | Length of data to be read |
|  | cb | Callback |
| Return value | Callback, val1 : the data read. val2 : The actual length read |  |
| Example code | SdkApi.getInstance().Hardware().flashRead( ApiTool.FlashRWType.APP_INFO, 1, new ResultCallback<ResultInfo<Interger>>() { @Override public void onMsg(ResultInfo<Integer> val) { byte[] array = val.getArrayByte(); int len = val.getVal(); } }); |  |
| Remark | Read type Data blocks correspond to the data at the specified address; data blocks are typically up to 1024 bytes in size. byte, some type Only a few bytes |  |

  - void flashWriteAddr(int type, int addr, int len, int[] buffer, ResultCallback<ResultInfo<Boolean>> cb)

Description: Writing Flash data

| type | describe |  |
| --- | --- | --- |
| parameter | type | Data type (HWID, (Authorization information, etc. ) ApiTool.FlashRWType kind |

|  | addr | storage address |
| --- | --- | --- |
|  | len | Write data length |
|  | buffer | Write data cache |
| Return value | true : Success false : failure |  |
| Example code | SdkApi.getInstance().Hardware().flashWrite( ApiTool.FlashRWType.APP_INFO, 0, 1, buffer new ResultCallback<ResultInfo<Boolean>>() { }); |  |
| Remark |  |  |

  - int mcuRead(int reg, byte[] buf, int len )

Description: Reading MCU Register data

| type | describe |  |
| --- | --- | --- |
| parameter | reg | Register address ( see MCU) (Communication Protocol ) |
|  | buf | Read data cache |
|  | len | Read length |
| Return value | Actual length read |  |
| Example code | int ret = SdkApi.getInstance().Hardware().mcuRead(reg,buf,len); |  |
| Remark | This interface is generally for internal use and is not publicly accessible. |  |

  - int mcuWrite(int reg, byte[] buf, int len )

Description: Modify MCU Register data

| type | describe |  |
| --- | --- | --- |
| parameter | reg | Register address ( see MCU) (Communication Protocol ) |
|  | buf | Write data cache |
|  | len | Write length |
| Return value | Actual length written |  |
| Example code | int ret = SdkApi.getInstance().Hardware().mcuRead(reg,buf,len); |  |
| Remark | This interface is generally for internal use and is not publicly accessible. |  |

  - int canBusConfigUpByID(int id, int bitrate)

Description: Set a specified ID baud rate of the device ( can )

| type | describe |  |
| --- | --- | --- |
| parameter | id | Can Device ID |
|  | bitrate | baud rate 9600 115200 |
| Return value | Set the return result for success or failure. |  |
| Example code | int ret = SdkApi.getInstance().Hardware().canBusConfigUpByID(id,bitrate); |  |
| Remark | Multiple Can To open the device settings, 3 steps are required. Second interval |  |

- Other related APIs

  - String getSimICCID()

Description: Obtain Sim Card ICCID number

| type | describe |  |
| --- | --- | --- |
| parameter |  |  |
| Return value | val: iccid |  |
| Example code | String iccid = SdkApi.getInstance().Other().getSimICCID(); |  |
| Remark | 4G connection required Modules and insert SIM Card |  |

  - String getModemIMEI()

Description: Get IMEI number

| type | describe |  |
| --- | --- | --- |
| parameter |  |  |
| Return value | val: imei |  |
| Example code | String imei = SdkApi.getInstance().Other().getModemIMEI(); |  |
| Remark | 4G connection required Modules and insert SIM Card |  |

  - void execShellCommand(String cmd)

Description: Execute shell Order

| type | describe |  |
| --- | --- | --- |
| parameter | cmd | shell Commands; for example: "input" keyevent 26” |
| Return value | void |  |
| Example code | SdkApi.getInstance().Other().execShellCommand(”input keyevent 26”); |  |
| Remark |  |  |

  - String getCTEI()

Description: Obtain CTEI Number

| type | describe |  |
| --- | --- | --- |
| parameter |  |  |
| Return value | val: ctei |  |
| Example code | String ctei = SdkApi.getInstance().Other().getCTEI(); |  |
| Remark | CTEI needs to be burned . Number |  |

  - String getEmmcLeftTime()

Description: Reading eMMC life

| type | describe |  |
| --- | --- | --- |
| parameter |  |  |
| Return value | val: leftTime; "010101" |  |
| Example code | String leftTime = SdkApi.getInstance().Other().getEmmcLeftTime(); |  |
| Remark |  |  |

- Scale Related APIs

  - int registerScaleListener(int appId, ScaleListener listener)

Description: Register scale Listener​

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.ScaleAppId |
|  | listener | Listener​ |
| Return value | Whether registration was successful, ApiTool.API_RES_* |  |
| Example code | int registerScaleListener(ApiTool.ScaleAppId.ID_APP_CUSTOM, new ScaleListener() { }); |  |
| Remark | Some motherboards do not have scale The chip is not supported, so it will not support this interface. |  |

  - int unregisterScaleListener(int appId, ScaleListener listener)

Description: Deregister scale Listener​

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.ScaleAppId |
|  | listener | Listener​ |
| Return value | Whether the cancellation was successful, ApiTool.API_RES_* |  |
| Example code | int unregisterScaleListener(ApiTool.ScaleAppId.ID_APP_CUSTOM, new ScaleListener() { }); |  |
| Remark | Some motherboards do not have scale The chip is not supported, so it will not support this interface. |  |

  - int setScaleEvent(int appId, int actionId, String strParam, int[] dataArray, int arrayLen)

Description: Sets the scale execution event.

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.ScaleAppId |
|  | actionId | The event ID is defined in ApiTool.ScaleEvent. |
|  | strParam | Parameters can be null . |
|  | dataArray | The parameter can be null ; when null , arrayLen... Must be 0 |
|  | arrayLen | dataArray Data length |
| Return value | Whether the setup was successful or not, the event result is in the listener. notify |  |
| Example code | int setScaleEvent(ApiTool.ScaleAppId.ID_APP_CUSTOM, ApiTool.ScaleEvent.QUERY_VERSION); |  |
| Remark | Some motherboards do not have scale The chip is not supported, so it will not support this interface. |  |

  - actionId command set

| command name | command code | illustrate | Remark |
| --- | --- | --- | --- |
| SCALE_EVENT_REPORT_SCREEN_TYPE | 0x300D | Reporting screen parameter index | For all registered Application callback |
| SCALE_EVENT_REPORT_SCREEN_VOLUME | 0x300E | Report screen brightness | For all registered Application callback |

示例

接口

Reporting screen parameter index

ScaleListener scaleListener = new ScaleListener() { @Override

public void onNotify(int actionId, byte[] dataArray, int dataLen) { switch (actionId) {

case SCALE_EVENT_REPORT_SCREEN_TYPE:

//todo your things break;

}

}

};

方法说明：重写 ScaleListener 回调接口，调用 registerScaleListener

返回参数说明：

actionId ：SCALE_EVENT_REPORT_SCREEN_TYPE

dataArray ：返回数据的数组[长度为 1byte]具体值待定义

部 分主 板没 有 scale芯 片 ，所 以会 不支 持该 接口

    - Report screen brightness

| interface | Example |
| --- | --- |
| Some motherboards do not have scale core Therefore, it will not support this interface . | ScaleListener scaleListener = new ScaleListener() { @Override public void onNotify(int actionId, byte[] dataArray, int dataLen) { switch (actionId) { case SCALE_EVENT_REPORT_SCREEN_VOLUME: //todo your Things break; } } }; Method Description: Override ScaleListener Callback interface, call registerScaleListener Return parameter description: actionId : SCALE_EVENT_REPORT_SCREEN_VOLUME dataArray : Returns an array of data [length is 1 byte ] [0x00 ] - [ 0x64 ] Volume value 0~100 |

- CommonEvent Related APIs

  - int registerCommonListener(int appId, ISdkCommonListener listener)

Description: Register Common Listener​

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.CommonAppId |
|  | listener | Listener​ |
| Return value | Whether registration was successful, ApiTool.API_RES_* |  |
| Example code | int registerCommonListener(ApiTool.CommonAppId.ID_APP_CUSTOM, new ISdkCommonListener() { }); |  |
| Remark |  |  |

  - int unregisterCommonListener(int appId, ISdkCommonListener listener)

Description: Unregister Common Listener​

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.CommonAppId |
|  | listener | Listener​ |
| Return value | Whether the cancellation was successful, ApiTool.API_RES_* |  |
| Example code | int unregisterCommonListener(ApiTool.CommonAppId.ID_APP_CUSTOM, listener); |  |
| Remark |  |  |

  - int setCommonEvent(int appId, int actionId, String strParam, int[] dataArray, int arrayLen)

Description: Set Common Execution Event

| type | describe |  |
| --- | --- | --- |
| parameter | appId | app Type ID , defined in ApiTool.CommonAppId |
|  | actionId | The event ID is defined in ApiTool.CommonEvent. |
|  | strParam | Parameters can be null . |
|  | dataArray | Parameters, spare parameters. If not used, please initialize to an empty array. Do not use this parameter. null​ |
|  | arrayLen | dataArray Data length |
| Return value | 11.4 Whether the setup was successful or not, the event result is in the listener. Notice, details can be found on 11.4 ISdkCommonListener Interface Description |  |
| Example code | int setCommonEvent(ApiTool.CommonAppId.ID_APP_CUSTOM, ApiTool.CommonEvent.EVENT_GET_LN); |  |
| Remark |  |  |

  - ISdkCommonListener Interface Description

| type | describe |  |
| --- | --- | --- |
| Listening parameters | actionId | The event ID is defined in ApiTool.CommonEvent. and |
|  | sVal | String , the parameter returned by the API, or the result of the API execution. |
|  | iArray | int An array, generally without special meaning, is used as a backup to return an array, which may assist in... intArrayLen The parameters are returned together as the result of the system interface operation. |
|  | intArrayLen | iArray Data length, the length received at this point The length is actually the interface. The success or failure status of the callback execution, and even the error code when it fails. |
| Remark | The callback is setCommonEvent Triggered after execution, making it convenient to receive interfaces with return values. |  |

  - CommonEvent Usage process

  - Example of calling the interface

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, actionId, params, dataArray, dataArray.length()); Parameter description: AppId : The process ID of the calling app; actionId : The unique ID of the function being called. params : Input parameters: If there are multiple parameters, params represents the JSON string. dataArray : Optional parameters; if not used, initialize to an empty array, and do not set it to null. dataArray.length() |

  - actionId command set

| command name | command code | illustrate | Remark |
| --- | --- | --- | --- |
| EVENT_GET_LT_VERSION | 0x1003 | Get the Longxun chip version |  |
| SWITCH_WIFI | 0x9000 | WiFi function switch |  |
| GET_WIFI_MAC | 0x9001 | Get WiFi MAC |  |
| SWITCH_BLUETOOTH | 0x9010 | Bluetooth function switch |  |
| GET_BLUETOOTH_MAC | 0x9011 | Get Bluetooth MAC |  |
| SWITCH_ETH | 0x9020 | Ethernet function switch |  |
| SWITCH_AP | 0x9030 | AP Function switch |  |
| GET_ETH_STATUS | 0x9040 | Get Ethernet status |  |
| SET_ETH_IP | 0x9050 | Configure Ethernet IP |  |
| GET_ETH_INFO | 0x9060 | Obtaining network information |  |
| GET_AP_INFO | 0x9070 | Get AP information |  |
| SET_SCREEN_SATURATION | 0x9080 | Set screen saturation |  |
| GET_SCREEN_SATURATION | 0x9081 | Get screen saturation |  |
| SET_SCREEN_CONTRAST | 0x9082 | Set screen contrast |  |

| GET_SCREEN_CONTRAST | 0x9083 | Get screen contrast |  |
| --- | --- | --- | --- |
| SET_SCREEN_HUE | 0x9084 | Set screen color tone |  |
| GET_SCREEN_HUE | 0x9085 | Get screen color tone |  |
| UPDATE_SYSTEM_LOGO | 0x9090 | Update system boot logo |  |
| UPDATE_SYSTEM_OTA | 0x9091 | OTA upgrade |  |
| SET_SCREEN_SIGNAL_BRIGHTNESS | 0x9086 | Set screen signal brightness |  |
| GET_SCREEN_SIGNAL_BRIGHTNESS | 0x9087 | Get screen signal brightness |  |
| EVENT_CONFIG_WIEGAND_DATA | 0x10004 | Wiegand configuration |  |
| EVENT_SEND_WIEGAND_DATA | 0x10005 | Wiegand sends data |  |
| EVENT_READ_WIEGAND_DATA | 0x10006 | Wiegand receives data |  |

    - SWITCH_WIFI ( 0x9000 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SWITCH_WIFI , params, dataArray, dataArray.length()); Parameter description: params: -- on : Open -- off : Turn off |

    - GET_WIFI_MAC ( 0x9001 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_WIFI_MAC , params, dataArray, dataArray.length()); Parameter description: params : None |

    - SWITCH_BLUETOOTH ( 0x9010 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_BLUETOOTH_MAC , params, dataArray, dataArray.length()); Parameter description: params: -- on : Open -- off : Turn off |

    - SWITCH_ETH ( 0x9020 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SWITCH_ETH , params, dataArray, dataArray.length()); Parameter description: params: Option 1: -- on : Open -- off : Turn off Option 2: -- true : Open -- false : Close Option 3: JSON class{ String ethNum ; // Network interface name, such as: eth0 boolean flagSwitch ; // Function switch } |

    - SWITCH_AP ( 0x9030 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SWITCH_AP , params, dataArray, dataArray.length()); Parameter description: params: json class{ boolean switchONOff ; // Function switch String apName ; //AP Name (boolean) apIp ; // Specifies the IP address String password ; // AP password (boolean) offWhenNoDeviceConn; /* * Security Type * SECURITY_TYPE_OPEN = 0 *SECURITY_TYPE_WPA2_PSK = 1 *SECURITY_TYPE_WPA3_SAE_TRANSITION = 2 *SECURITY_TYPE_WPA3_SAE = 3 / private int pwType; |

|  | /* * bandwidth * BAND_2GHZ = 1 << 0 * BAND_5GHZ = 1 << 1 *BAND_6GHZ = 1 << 2 *BAND_60GHZ = 1 << 3 / private int band } |
| --- | --- |

    - GET_ETH_STATUS ( 0x9040 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_ETH_STATUS , params, dataArray, dataArray.length()); Parameter description: params : Network driver name (iface) , such as eth0 |
| Return value | sVal: -- true : Open -- false : Close |

    - SET_ETH_IP ( 0x9050 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SET_ETH_IP , params, dataArray, dataArray.length()); Parameter description: params: json class{ String iface ; // Network driver name, such as eth0 boolean isStatic ; // DHCP Static IP Pattern String ipAddr ; String mask ; String gate ; String dns1 ; String dns2 ; } |
| Return value | sVal: -- 0 : Success -- Other values: Failure |

    - GET_ETH_INFO ( 0x9060 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_ETH_INFO , params, dataArray, dataArray.length()); Parameter description: params : Network driver name (iface) , such as eth0 |
| Return value | sVal : Network driver information |

    - GET_AP_INFO ( 0x9070 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_AP_INFO , params, dataArray, dataArray.length()); Parameter description: params: null |
| Return value | sVal: AP driver information |

    - SET_SCREEN_SATURATION ( 0x9080 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SET_SCREEN_SATURATION , params, dataArray, dataArray.length()); Parameter description: dataArray[0]: 0 - 100 |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - GET_SCREEN_SATURATION ( 0x9081 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_SCREEN_SATURATION , params, dataArray, dataArray.length()); Parameter description: params: null |
| Return value | sVal: 0 - 100 iArray[0]： -- 0 : Success -- - 1 : Failure |

|  | --- 2 : Interface not supported |
| --- | --- |

    - SET_SCREEN_CONTRAST ( 0x9082 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SET_SCREEN_CONTRAST , params, dataArray, dataArray.length()); Parameter description: dataArray[0]: 0 - 100 |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - GET_SCREEN_CONTRAST ( 0x9083 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_SCREEN_CONTRAST , params, dataArray, dataArray.length()); Parameter description: params: null |
| Return value | sVal: 0 - 100 iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - SET_SCREEN_HUE ( 0x9084 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SET_SCREEN_HUE , params, dataArray, dataArray.length()); Parameter description: dataArray[0]: 0 - 100 |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - GET_SCREEN_HUE ( 0x9085 )

| interface | Example |
| --- | --- |

|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_SCREEN_HUE , params, dataArray, dataArray.length()); Parameter description: params: null |
| --- | --- |
| Return value | sVal: 0 - 100 iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - UPDATE_SYSTEM_LOGO ( 0x9090 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, UPDATE_SYSTEM_LOGO , params, dataArray, dataArray.length()); Parameter description: params: json class{ int type ; //0:animation 1:boot logo String orientation ; //horizontal: landscape vertical: portrait mode String logoFilePath ; // Absolute path to the file int logoFileType ; //0:png 1:zip 2:mp4 } |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - UPDATE_SYSTEM_OTA ( 0x9091 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, UPDATE_SYSTEM_OTA , params, dataArray, dataArray.length()); Parameter description: params: -- String: Absolute path to the OTA file, must be /mnt/sdcard/update.zip |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - UPDATE_SYSTEM_UBOOT_LOGO ( 0x9092 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, UPDATE_SYSTEM_UBOOT_LOGO, params, dataArray, dataArray.length()); Parameter description: params: -- String: Uboot The absolute path to the logo file; the file must be in .bmp format . Note: Requires an 8-bit or 24 -bit BMP image , bootable tbmp and bootable . The kernel.bmp images have the same size and resolution. |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - UPDATE_SYSTEM_KERNEL_LOGO 	( 0x9093 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, UPDATE_SYSTEM_KERNEL_LOGO , params, dataArray, dataArray.length()); Parameter description: params: -- String: kernel The absolute path to the logo file; the file must be in .bmp format . Note: Requires an 8-bit or 24 -bit BMP image , bootable tbmp and bootable . The kernel.bmp images have the same size and resolution. |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - SET_SCREEN_SIGNAL_BRIGHTNESS ( 0x9086 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, SET_SCREEN_SIGNAL_BRIGHTNESS, params, dataArray, dataArray.length()); Parameter description: |

|  | dataArray[0]: 0 - 100 |
| --- | --- |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - GET_SCREEN_SIGNAL_BRIGHTNESS ( 0x9087 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, GET_SCREEN_SIGNAL_BRIGHTNESS, params, dataArray, dataArray.length()); Parameter description: params: null |
| Return value | sVal: 0 - 100 iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - EVENT_CONFIG_WIEGAND ( 0x10004 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, EVENT_CONFIG_WIEGAND , params, dataArray, dataArray.length()); Parameter description: params: -- String : Wiegand bits , containing " 26", "32", "34", or "58" dataArray: -- int array: Requires initialization with 1 byte of data, containing 0. |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - EVENT_SEND_WIEGAND_DATA ( 0x10005 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, EVENT_SEND_WIEGAND_DATA , params, dataArray, dataArray.length()); Parameter description: |

|  | dataArray: -- int array: transmits fixed-size data based on Wiegand bits . -- Example: In 26 -bit mode, transmit 3 bytes of data: dataArray[3]={0x01 , 0x02 , 0x03} |
| --- | --- |
| Return value | iArray[0]： -- 0 : Success -- - 1 : Failure --- 2 : Interface not supported |

    - EVENT_READ_WIEGAND_DATA ( 0x10006 )

| interface | Example |
| --- | --- |
|  | SdkApi.getInstance().CommonEvent().setCommonEvent(AppId, EVENT_READ_WIEGAND_DATA , params, dataArray, dataArray.length()); Parameter description: dataArray: -- int array: Requires initialization with 1 byte of data, containing 0. Recommendation: Read Wiegand data every 500ms . |
| Return value | iArray[0]： -- 0 : Success >> strVal : Returns the data read; otherwise, returns failure. -- - 1 : Failure --- 2 : Interface not supported |